TCDC District Plan

#### 453 - Standards

Floor levels of all houses and all habitable rooms shall meet the following standards:

In areas covered by flood management plans:

| (a) | Primary overland flow areas: Not less than one metre above natural ground level; |
| --- | --- |
| (b) | Secondary overland flow areas: Not less than 0.5 metres above natural ground level; |
| (c) | Ponding areas: Not less than 0.5m above the flood datum level stated on the planning map; |
| (d) | Overland flow and ponding areas: Not less than one metre above natural ground level. |

Floor levels of all houses and all habitable rooms for sites located in a flood hazard area shall meet the following standard;
Not less than 0.5 metres above predicted flood levels. Predicted flood levels are determined by reference to flood modelling, flooding history, a derived flood event, and existing flood protection measures. In defended areas, floor levels must be 0.5 metres above the predicted flood level for a 1% flood event.

In defended areas, new houses must be set back a minimum of 20 metres from the base of any flood defence, unless an easement, or other legal instrument, for the purposes of access to and maintenance of the flood defence has been registered on the Computer Freehold Register or Certificate of Title.

In medium flood hazard areas, replacement houses or additions to houses must be designed with foundations that are open and allow the free passage of floodwaters to pass beneath them to ensure habitable areas are not subject to inundation and floodwaters are not diverted or displaced onto surrounding properties.

To ensure compliance with this standard a report from a suitably qualified expert (e.g. hydrological engineer) is required to accompany any building consent for the activities listed above, confirming that the proposal complies with this requirement.

| Note | Ground levels and hazard setback lines should be confirmed by survey. |
| --- | --- |
| Note | Coastal flooding and erosion areas setback standards are applied and enforced under the Building Act. Locations where the setbacks apply are shown on the same maps as those which define "Beachfront Yards" in Section 9 under "Yard". The setbacks under the Building Act apply to buildings and structures including septic tanks and their disposal fields as follows: No Buildings: Structures or septic tanks or their disposal fields shall be located within 30m inland from the toe of the natural shoreline or seaward vegetated toe of the foredune on the East Coast. No Buildings: Structures or septic tanks or their disposal fields shall be located within 15m inland from the natural shoreline on the West Coast between Tararu and Waikawau (inclusive). Relocated Buildings: May be situated between the 30m and 60m hazard lines, East Coast (measured as above), provided that Council may require a greater setback or building to be relocatable in other circumstances if required because of the severity of the erosion or flooding risk. |
| Note | Ground levels and hazard setback lines should be confirmed by the most appropriate means for the solution as directed by Council's Building Inspector. |